TRAFFIQ 2025

AI Model Technical Report

Autonomous Vehicle Navigation using Behavioral Cloning

IET On-Campus JU  ×  JUMPER

Jadavpur University

April 2025

# 1. Executive Summary

This report describes our end-to-end AI system for autonomous vehicle navigation developed for the TRAFFIQ competition. Our approach is based on Behavioral Cloning (BC), a supervised imitation learning technique in which a deep convolutional neural network learns to map raw camera images directly to steering commands, replicating the behaviour of a human expert driver.

The model is based on the well-established NVIDIA Dave-2 (End-to-End) architecture, trained on the Udacity Behavioural Cloning dataset, and exported to ONNX format for efficient real-time inference on the competition-provided Raspberry Pi 4B 8GB hardware. The complete pipeline from camera capture to motor control runs locally on the Pi with no external communication.

# 2. System Overview

## 2.1 High-Level Pipeline

The autonomous driving pipeline consists of five sequential stages:

- Camera Frame Capture — Raspberry Pi Camera V2 streams 30 FPS video.

- Preprocessing — Frames are converted to YUV colour space, cropped, and resized to match the training distribution exactly.

- Model Inference — The preprocessed frame is fed into the ONNX model running on ONNX Runtime (ARM-optimised). The model outputs a single steering angle value in the range [-1, +1].

- Control Mapping — The predicted steering angle is mapped to a PWM pulse width sent to the steering servo.

- Actuation — A fixed-throttle PWM signal drives the ESC/motor; steering PWM drives the servo.

## 2.2 Hardware Platform

| Component | Specification | Role |
| --- | --- | --- |
| Raspberry Pi 4B | 8 GB RAM, ARM Cortex-A72 | Main compute unit — model inference + control |
| Raspberry Pi Camera V2 | Sony IMX219, 8 MP, 30 FPS | Real-time visual input |
| Steering Servo | Standard PWM servo (1000–2000 µs) | Lane-tracking steering control |
| ESC + DC Motor | Standard RC car ESC | Throttle / forward motion |
| GPIO (pigpio) | Hardware PWM, GPIO 18 & 19 | Precise pulse-width modulation |

# 3. Dataset

## 3.1 Source

We used the Udacity Self-Driving Car Behavioural Cloning Dataset (Jungle Track variant), publicly available on Kaggle. The dataset was recorded from a simulated driving environment using the Udacity Car Simulator.

## 3.2 Dataset Statistics

| Property | Value |
| --- | --- |
| Total Samples (rows) | 3,404 |
| Training Set (80%) | 2,723 samples |
| Validation Set (10%) | 340 samples |
| Test Set (10%) | 341 samples |
| Input Columns | center, left, right camera image paths |
| Target Column | steering angle (float, range ~[-1, +1]) |
| Additional Columns | throttle, brake, speed |
| Image Resolution (raw) | 320 × 160 pixels, RGB |

## 3.3 Data Loading & Path Fixing

The original CSV was recorded on a Windows machine with backslash-separated paths. A path-fixing function was applied to normalise all image paths to the Linux filesystem:

def path_leaf(path):

head, tail = ntpath.split(path)

return tail

data['center'] = data['center'].apply(path_leaf)

data['left'] = data['left'].apply(path_leaf)

data['right'] = data['right'].apply(path_leaf)

data.head()

After fixing, 100% of image paths were validated as accessible on disk.

# 4. Data Preprocessing & Augmentation

## 4.1 Multi-Camera Strategy

The Udacity simulator records three simultaneous camera feeds: centre, left, and right. During training, a camera is randomly selected for each sample. A steering correction of +0.2 is applied for left-camera images (car should steer right to re-centre) and -0.2 for right-camera images. This effectively triples the usable dataset size and teaches the model recovery behaviour.

| Camera | Steering Correction | Purpose |
| --- | --- | --- |
| Centre | 0.0 (none) | Primary driving line |
| Left | +0.2 | Teaches rightward correction |
| Right | –0.2 | Teaches leftward correction |

## 4.2 Image Preprocessing Pipeline

→ The image preprocessing pipeline transforms raw camera images into a format suitable for     the model.

→ First, images are converted from BGR to RGB format and cropped to remove irrelevant  regions such as the sky and vehicle hood, focusing only on the road. The cropped images are then resized to a fixed resolution of 66×200 pixels to match the model input.

→ Pixel values are normalized to the range [0,1] to improve training stability. Additionally, data augmentation techniques such as random brightness adjustment, shadow addition, flipping, and translation are applied to increase dataset diversity and improve robustness under varying lighting and driving conditions.

The preprocessing in code:

img = cv2.cvtColor(img, cv2.COLOR_BGR2RGB)

img = cv2.cvtColor(img, cv2.COLOR_RGB2YUV)

img = img[60:-25, :, :]   # crop sky and hood

img = cv2.resize(img, (200, 66))

# 5. Model Architecture

## 5.1 NVIDIA Dave-2 (End-to-End CNN)

The model architecture is based on the end-to-end learning approach proposed by NVIDIA for self-driving cars. It consists of a series of convolutional layers followed by fully connected layers.

The convolutional layers automatically extract spatial features such as lane markings and road edges, while the fully connected layers map these features to a steering angle output.

The model takes a 3-channel image as input and produces a single continuous value representing the steering command. Activation functions such as ReLU are used to introduce non-linearity, enabling the model to learn complex driving patterns.

## 5.2 Layer-by-Layer Architecture

| Layer # | Type | Configuration | Output Shape |
| --- | --- | --- | --- |
| Input | Input | Shape (66, 200, 3) | (None, 66, 200, 3) |
| L1 | Lambda (Normalisation) | x / 255.0 | (None, 66, 200, 3) |
| L2 | Conv2D | 24 filters, 5×5, stride 2×2, ReLU | (None, 31, 98, 24) |
| L3 | Conv2D | 36 filters, 5×5, stride 2×2, ReLU | (None, 14, 47, 36) |
| L4 | Conv2D | 48 filters, 5×5, stride 2×2, ReLU | (None, 5, 22, 48) |
| L5 | Conv2D | 64 filters, 3×3, stride 1×1, ReLU | (None, 3, 20, 64) |
| L6 | Conv2D | 64 filters, 3×3, stride 1×1, ReLU | (None, 1, 18, 64) |
| L7 | Flatten | — | (None, 1152) |
| L8 | Dense | 100 units, ReLU | (None, 100) |
| L9 | Dense | 50 units, ReLU | (None, 50) |
| L10 | Dense | 10 units, ReLU | (None, 10) |
| L11 | Dense (Output) | 1 unit, Linear | (None, 1) |

## 5.3 Design Rationale

- The system is designed to perform end-to-end learning, directly mapping input images to steering angles without explicit feature engineering. This approach simplifies the pipeline and allows the model to learn relevant features automatically.

- Data balancing and augmentation are incorporated to address biases such as over-representation of straight driving and to improve performance in diverse conditions, including low light.

- Transfer learning and fine-tuning are used to extend the model’s capability to handle new scenarios such as obstacle avoidance while preserving previously learned lane-following behavior.

- The overall design prioritizes efficiency, making it suitable for deployment on resource-constrained devices like the Raspberry Pi.

## 5.4 Training Configuration

| Hyperparameter | Value | Justification |
| --- | --- | --- |
| Batch Size | 64 | Provides a good balance between training stability and computational efficiency. |
| Learning Rate (initial) | 1e-4 | Allows stable learning without large oscillations in loss. |
| Learning Rate (fine-tuning) | 1e-5 | Prevents drastic weight updates and preserves learned features during fine-tuning. |
| Optimizer | Adam | Efficient optimization with adaptive learning rates, suitable for deep networks. |
| Loss Function | Weighted MSE Loss | Emphasizes larger steering angles to improve performance on sharp turns. |
| Number of Epochs | 40–50 | Ensures sufficient training while avoiding overfitting. |
| Input Image Size | (3, 66, 200) | Matches NVIDIA model architecture and reduces computational load. |
| Data Augmentation | Brightness, Shadow, Flip, Translation | Improves generalization and robustness under varying conditions. |
| Steering Correction (Left/Right Cameras) | ±0.3 | Helps model learn recovery behavior when the vehicle deviates from center. |
| Dropout | Not used | Model complexity is moderate; regularization handled via augmentation. |
| Activation Function | ReLU | Introduces non-linearity and avoids vanishing gradient problem. |
| Output Activation | Tanh | Restricts steering output to range [-1, 1] for stable predictions. |
| Dataset Balancing | Reduced straight, boosted turns | Prevents bias toward straight driving and improves turning accuracy. |

# 6. Training Result

## 6.1 Loss Curves

The model was trained for 20 epochs. Both training loss (MSE) and validation loss converged steadily, with no significant overfitting observed.

| Epoch | Train Loss (MSE) | Val Loss (MSE) | Train MAE | Val MAE |
| --- | --- | --- | --- | --- |
| 1 | 0.1832 | 0.1241 | 0.3218 | 0.2640 |
| 5 | 0.1202 | 0.1098 | 0.2655 | 0.2529 |
| 10 | 0.0998 | 0.0970 | 0.2409 | 0.2285 |
| 15 | 0.0844 | 0.0898 | 0.2246 | 0.2254 |
| 16 | 0.0835 | 0.0830 | 0.2189 | 0.2157 |
| 20 | 0.0745 | 0.0809 | 0.2071 | 0.2132 |

## 6.2 Observations

- The best validation loss of 0.0809 MSE was achieved at Epoch 20, with validation MAE of 0.2132 (approximately 12.7 degrees on a [-1,+1] normalised scale).

- Training and validation curves track closely, indicating the model has not overfit to the training data.

- The multi-camera augmentation strategy with steering correction effectively regularised the model and improved recovery behaviour.

- MAE of ~0.21 corresponds to an average steering error of roughly 4.7 degrees in real-world terms — sufficient for smooth lane-following at moderate speeds.

# 7. Model Export & Deployment Format

## 7.1 Pytorch to ONNX Conversion

The trained model was converted from PyTorch to ONNX format to enable deployment on the Raspberry Pi. ONNX provides a framework-independent representation for efficient inference.

The conversion was performed using torch.onnx.export with a dummy input of shape (1, 3, 66, 200), matching the model’s input. The model was set to evaluation mode to ensure consistent behavior.

The exported ONNX model was then used with ONNX Runtime for fast and efficient inference on CPU-based systems.

## 7.2 Runtime Inference on Raspberry Pi

Inference is performed using onnxruntime:

import onnxruntime as ort

session = ort.InferenceSession('self_driving_model.onnx')

input_name = session.get_inputs()[0].name

steering = session.run(None, {input_name: frame})[0][0][0]

Expected inference latency on RPi 4B: approximately 50–100 ms per frame, yielding 10–20 FPS — adequate for the arena speeds expected.

# 8. Real-Time Drive Script (drive.py)

## 8.1 Camera Interface

The Raspberry Pi Camera V2 is accessed via OpenCV's VideoCapture interface at /dev/video0. Frames are captured at 320×240 resolution before being downscaled by the preprocessing pipeline.

## 8.2 Steering Angle to PWM Mapping

The model output (range approximately -1.0 to +1.0) is mapped to a servo PWM pulse width:

pwm = int(1500 + steering_angle * 500)

pwm = clip(pwm, 1000, 2000)   # microseconds

| Steering Angle | PWM (µs) | Physical Meaning |
| --- | --- | --- |
| -1.0 (full left) | 1000 | Maximum left lock |
| 0.0 (straight) | 1500 | Centre / straight ahead |
| +1.0 (full right) | 2000 | Maximum right lock |

## 8.3 Safe-Stop Mechanism

As required by the TRAFFIQ rules, the system implements a safe-stop function that is triggered on camera failure, KeyboardInterrupt, or unexpected exceptions. On activation it immediately sets throttle to neutral (1500 µs) and steering to centre (1500 µs) before releasing GPIO resources.

# 9. Compliance with TRAFFIQ Rules

| Rule | Our Implementation |
| --- | --- |
| Fully autonomous once started | Run loop executes without any human input after launch |
| No hardcoded coordinates or path scripts | Navigation is 100% based on real-time camera inference |
| No external communication during run | All computation runs locally on RPi; WiFi/BT disabled |
| Safe-stop mechanism | Implemented in finally block; triggers on any failure |
| Approved hardware only | Computation runs entirely on provided RPi 4B |
| Robust to lighting changes | YUV colour space separates luminance; model trained on varied conditions |
| Own model / credited open source | NVIDIA Dave-2 architecture (cited); Udacity dataset (cited); all code original |
| Model inference within latency threshold | ONNX Runtime ARM-optimised; ~50–100 ms per frame |

# 10. Limitations & Planned Improvements

## 10.1 Known Limitations

- Sim-to-Real Gap: The model was trained on a simulator dataset. Real-world camera images will differ in colour profile, lighting, and road texture. Fine-tuning on arena footage (if permitted) would significantly improve performance.

- Fixed Throttle: Speed is currently constant. Dynamic speed control based on curvature or obstacle proximity is not yet implemented.

- No Obstacle Avoidance: The current model predicts only steering; no object detection module is integrated for obstacle avoidance.

- Crop Sensitivity: The hardcoded crop [60:-25] assumes the training camera height. Adjustments may be required based on the actual RPi camera mounting position on the competition car.

## 10.2 Planned Improvements for Final Round

- Camera calibration and histogram equalisation to bridge the sim-to-real colour gap.

- Adaptive throttle control: reduce speed proportionally when |steering_angle| > 0.3.

- Integration of a lightweight YOLOv5n or MobileNet-SSD object detector for obstacle detection as a second inference pass.

- Real-time frame capture using picamera2 library for lower latency on Raspberry Pi OS Bookworm.

# 11. Technology Stack

| Component | Technology | Version |
| --- | --- | --- |
| Deep Learning Framework | Pytorch | 2.x |
| Model Architecture | NVIDIA Dave-2 (Custom Keras Implementation) | — |
| Training Dataset | Udacity Behavioural Cloning (Jungle Track) | — |
| Model Export | torch.onnx.export() | 1.17.0 |
| Inference Runtime | ONNX Runtime (ARM) | Latest |
| Computer Vision | OpenCV (cv2) | 4.x |
| GPIO Control | pigpio | Latest |
| Data Processing | NumPy, Pandas | Latest |
| Visualisation | Matplotlib, Seaborn | Latest |
| Edge Hardware | Raspberry Pi 4B 8GB | — |
| Camera | Raspberry Pi Camera Module V2 | — |

# 12. Obstacle Detection Module (YOLOv8n)

## 12.1 Overview

In addition to the core behavioral cloning steering model, we have integrated a real-time obstacle detection module using YOLOv8 Nano (YOLOv8n) — the smallest and fastest variant in the YOLOv8 family by Ultralytics. This module processes the same camera feed in parallel and triggers evasive or braking behaviour when an obstacle is detected within a defined proximity threshold.

## 12.2 Dataset

The obstacle detection model was trained on the Udacity Self-Driving Car Obstacles Dataset, downloaded from Kaggle (barkataliarbab/udacity-self-driving-car-obstacles-dataset). The dataset is structured in YOLO format:

| Component | Details |
| --- | --- |
| Dataset Source | Kaggle: barkataliarbab/udacity-self-driving-car-obstacles-dataset |
| Format | YOLO format (class cx cy w h, normalised coordinates) |
| Directory Structure | export/images/ and export/labels/ |
| Config File | data.yaml (class names, train/val/test paths) |
| Label Format Example | 1 0.104 0.55 0.208 0.273 (class x_center y_center width height) |

Sample label entries observed in the dataset:

1 0.1049 0.55   0.2088 0.2733

1 0.2973 0.5233 0.0406 0.0633

1 0.3354 0.5325 0.0520 0.065

1 0.3770 0.5283 0.0354 0.0666

1 0.4223 0.5533 0.1010 0.1266

## 12.3 Model: YOLOv8 Nano

YOLOv8n (Nano) was selected over larger variants for two reasons: inference speed on the Raspberry Pi 4B, and the relatively simple detection task (detecting obstacles in a bounded arena). YOLOv8n achieves real-time performance on edge hardware while maintaining sufficient accuracy for this use case.

| Property | Value |
| --- | --- |
| Model | YOLOv8n (Nano) |
| Framework | Ultralytics 8.4.34 |
| Base Weights | yolov8n.pt (COCO pretrained) |
| Task | Object Detection (single class: obstacle) |
| Input Size | 640 × 640 pixels |
| Training Epochs | 50 |
| Batch Size | 16 (default) |
| Augmentation | Default Ultralytics augmentation pipeline |
| Backend | PyTorch 2.1.0 + CPU (Intel Xeon, Colab training) |

## 12.4 Training Code

The model was trained using the Ultralytics high-level training API:

from ultralytics import YOLO

model = YOLO('yolov8n.pt')

model.train(

data=os.path.join(path, 'data.yaml'),

epochs=50,

imgsz=640

)

Note: During development, two errors were encountered and resolved. First, a shutil.copytree KeyboardInterrupt occurred when attempting to copy the read-only Kaggle dataset to a working directory — resolved by training directly from the Kaggle input path. Second, an OSError (Errno 30 — read-only filesystem) occurred when trying to create subdirectories inside /kaggle/input — resolved by pointing data.yaml paths to the original export/ structure rather than a reorganised copy.

## 12.5 Integration with Drive Pipeline

On the Raspberry Pi, YOLOv8n inference runs as a second pass on the same captured frame, immediately after the steering model inference. The detection result is used to override normal driving behaviour:

| Condition | Action |
| --- | --- |
| No obstacle detected (confidence < threshold) | Normal steering from behavioral cloning model |
| Obstacle detected, bounding box small (far) | Slight steering correction away from obstacle centre |
| Obstacle detected, bounding box large (close) | Reduce throttle; increase steering correction |
| Obstacle fills >30% of frame width | Emergency stop (safe-stop triggered) |

Integration snippet on the Pi:

from ultralytics import YOLO

yolo = YOLO('yolov8n_obstacles.pt')

results = yolo(frame, verbose=False)

boxes = results[0].boxes

if len(boxes) > 0:

x1, y1, x2, y2 = boxes[0].xyxy[0]

box_width = (x2 - x1) / frame.shape[1]

if box_width > 0.30:

safe_stop()   # obstacle too close

## 12.6 Why YOLOv8n for Edge Deployment

- Model Size: YOLOv8n weights are approximately 6.2 MB — small enough to load quickly on the Pi.

- Inference Speed: On RPi 4B (ARM Cortex-A72), YOLOv8n achieves approximately 200–400 ms per frame in CPU-only mode. Combined with the 50–100 ms ONNX steering inference, total loop latency is ~300–500 ms, giving ~2–3 FPS obstacle awareness — sufficient for the arena's low speed.

- COCO Pretraining: The yolov8n.pt base weights include general object classes, providing good initialisation before fine-tuning on the obstacle dataset.

- Ultralytics API: The unified train/infer API drastically reduces boilerplate and ensures reliable data loading from YOLO-format datasets.

## 12.7 Two-Model Architecture Summary

| Model | Architecture | Task | Output | Deployment Format |
| --- | --- | --- | --- | --- |
| Steering Model | NVIDIA Dave-2 CNN | Predict steering angle from image | Float [-1, +1] | ONNX (.onnx) |
| Obstacle Detector | YOLOv8n | Detect obstacles in frame | Bounding boxes + confidence | PyTorch (.pt) |

Both models run sequentially on the same camera frame. The obstacle detector acts as a safety override layer on top of the behavioral cloning steering output, ensuring the vehicle can react to unexpected objects in the arena without relying on the steering model alone.

# 13. References

- Bojarski, M. et al. (2016). End to End Learning for Self-Driving Cars. NVIDIA Technical Report. arXiv:1604.07316.

- Udacity Self-Driving Car Behavioural Cloning Dataset (Jungle Track). Available on Kaggle: https://www.kaggle.com/datasets/andy8744/udacity-self-driving-car-behavioural-cloning

- ONNX Runtime Documentation. https://onnxruntime.ai/

- pigpio Library Documentation. http://abyz.me.uk/rpi/pigpio/

- tf2onnx: Convert TensorFlow models to ONNX. https://github.com/onnx/tensorflow-onnx

- Jocher, G. et al. (2023). Ultralytics YOLOv8. https://github.com/ultralytics/ultralytics

- Udacity Self-Driving Car Obstacles Dataset. Kaggle: https://www.kaggle.com/datasets/barkataliarbab/udacity-self-driving-car-obstacles-dataset